RTL-to-GDSII Implementation of a

UART-to-I2C Bridge

Using Open-Source EDA Tools and the SKY130 PDK

A Complete, Reproducible Walkthrough

From Beginner Fundamentals to Industry-Grade Signoff

Toolchain: Icarus Verilog · GTKWave · Yosys · OpenLane · OpenROAD · Magic · KLayout · Netgen

Technology: SKY130A (130 nm) — sky130_fd_sc_hd standard cell library

Target Frequency: 100 MHz  |  Achieved: 99.8 MHz, 0 setup/hold violations

# Table of Contents

1. Introduction	3

2. Prerequisites and Tool Installation	3

3. Project Directory Setup	5

4. RTL Simulation with Icarus Verilog	6

5. Waveform Analysis using GTKWave	6

6. RTL Synthesis with Yosys	8

7. OpenLane Physical Design Flow (RTL-to-GDSII Steps)	9

8. Floorplanning Results	10

9. Placement Results	11

10. Clock Tree Synthesis (CTS) Results	11

11. Routing Results	12

12. Static Timing Analysis (STA) Results	12

13. Power Analysis	12

14. DRC and LVS Verification	14

15. GDSII Generation and Visualization	15

16. Conclusion and Future Work	17

Appendix A: Full Command Reference	18

Appendix B: Glossary (for beginners)	19

# 1. Introduction

Open-source Electronic Design Automation (EDA) has matured to the point where a complete digital ASIC — from a hardware description in Verilog down to a fabrication-ready GDSII layout — can be built without any commercial CAD license. This project is a full, worked example of that flow: a UART-to-I2C protocol bridge, taken all the way from RTL simulation through synthesis, physical design, and signoff verification on the SkyWater SKY130 open PDK.

### Concept: What is RTL-to-GDSII?

"RTL" (Register Transfer Level) is the behavioral Verilog code an engineer writes. "GDSII" is the binary geometry file sent to a semiconductor foundry for fabrication. The steps in between — synthesis, floorplanning, placement, clock-tree synthesis, routing, and signoff — are collectively called the "physical design flow", and are what turns an abstract description of logic into physical shapes on silicon.

The design converts asynchronous UART serial data into I2C bus transactions, and was implemented with the following fully open-source toolchain:

- Icarus Verilog — RTL simulation / compilation

- GTKWave — waveform viewing

- Yosys — RTL synthesis (logic → gate-level netlist)

- OpenLane — automated RTL-to-GDSII flow orchestration

- OpenROAD — floorplanning, placement, CTS, routing, STA engine used inside OpenLane

- Magic — layout view, DRC, and GDS handling

- KLayout — layout viewing and DRC/LVS cross-checking

- Netgen — LVS (layout-vs-schematic verification)

Technology node: SKY130A (130 nm CMOS), sky130_fd_sc_hd standard cell library.

# 2. Prerequisites and Tool Installation

These steps set up a fresh Ubuntu Linux machine (20.04/22.04 recommended) with every tool used in this guide. Beginners: run these once; you will not need to repeat them for future projects.

## 2.1 System Packages and Standalone Tools

Icarus Verilog, GTKWave, Yosys, Magic, and KLayout are all packaged for Ubuntu and can be installed directly:

sudo apt update
sudo apt install -y build-essential git python3 python3-pip \
    iverilog gtkwave yosys magic klayout

Installs the standalone simulation, synthesis, and layout-viewing tools.

Verify each tool installed correctly:

iverilog -V | head -n 1
gtkwave --version
yosys -V
magic --version
klayout -v

## 2.2 Docker (required by OpenLane)

OpenLane's classic flow runs inside a Docker container that bundles OpenROAD, Magic, Netgen, and the SKY130 PDK with matched, tested versions.

sudo apt install -y docker.io
sudo systemctl enable --now docker
sudo usermod -aG docker $USER
newgrp docker          # refresh group membership in this shell

## 2.3 OpenLane and the SKY130 PDK

git clone https://github.com/The-OpenROAD-Project/OpenLane.git
cd OpenLane
make            # builds/pulls the OpenLane Docker image
make pdk        # fetches and builds the SKY130 open PDK via Volare/open_pdks
make test       # runs a small built-in design to confirm the flow works

💡 `make test` should finish with a message confirming the sample design passed DRC, LVS, and timing checks. If it fails, re-check the Docker group membership step above (log out/in may be required).

# 3. Project Directory Setup

OpenLane expects each design to live in its own folder under designs/, containing the RTL source and a configuration file describing the design's name, clock, and target utilization.

cd OpenLane/designs
mkdir -p i2c_uart_bridge/src
cd i2c_uart_bridge

Place the seven RTL source files in src/, then create config.tcl:

set ::env(DESIGN_NAME) i2c_uart_bridge
set ::env(VERILOG_FILES) "dir::src/*.v"
set ::env(CLOCK_PORT) "clk"
set ::env(CLOCK_PERIOD) "10"
set ::env(FP_CORE_UTIL) 50
set ::env(PL_TARGET_DENSITY) 0.55
set ::env(SYNTH_STRATEGY) "AREA 0"

config.tcl — clock period of 10 ns targets the 100 MHz frequency confirmed in Section 11.

## 3.1 RTL Module Overview

| Module | Function |
| --- | --- |
| top.v | Top-level integration |
| uart_rx.v | UART receiver |
| uart_tx.v | UART transmitter |
| baud_gen.v | Baud rate generation |
| fifo.v | Temporary buffering |
| control_fsm.v | Protocol conversion logic |
| reg_file.v | Internal register storage |
| i2c_slave.v | I2C slave communication |

The UART receiver captures incoming serial frames and passes them to the control FSM, which interprets commands and drives the I2C slave interface accordingly.

# 4. RTL Simulation with Icarus Verilog

Before spending any time on physical design, the RTL must be verified functionally. Icarus Verilog compiles the design and its testbench into an executable simulation model.

cd designs/i2c_uart_bridge
iverilog -o sim.out src/*.v tb/*.v
vvp sim.out
gtkwave dump_top.vcd

### Concept: Testbench vs. Design

The testbench (files under tb/) is Verilog code that is never synthesized — it only exists to drive stimulus into the design and check its responses during simulation, producing a .vcd waveform dump for GTKWave.

Simulation confirmed:

- UART frame reception

- Register read/write operations

- I2C transaction generation

- Correct protocol conversion across the UART/I2C clock domains

# 5. Waveform Analysis using GTKWave

GTKWave renders the .vcd dump as timing waveforms so signal transitions can be inspected visually against expected protocol behavior.

Figure 1.  —  GTKWave view of dump_top.vcd showing UART capture (uart_cap), UART period, and I2C sda_bus/scl activity

Signals of interest for this design:

- UART RX / TX lines

- Start and stop bits

- Control FSM state transitions

- I2C SDA and SCL lines

- Internal register addresses and data bus activity

The waveform in Figure 1 confirms successful hand-off of data between the UART and I2C clock domains — the captured byte values (e.g. 0xB5, 0xA5, 0xC3) appear correctly on both the uart_cap register and are reflected onto the I2C bus.

# 6. RTL Synthesis with Yosys

Synthesis translates behavioral Verilog into a gate-level netlist built entirely from SKY130 standard cells. OpenLane calls Yosys internally during its first flow stage, but it can also be run standalone to inspect the resulting network of gates, shown in Figure 2.

yosys -p "synth -top top; write_verilog synth_top.v; show -format dot -prefix yosys_show" src/*.v
xdot yosys_show.dot        # opens the interactive gate-level graph viewer

Figure 2.  —  Synthesized gate-level netlist graph for the UART-to-I2C bridge, viewed with XDot

### Synthesis Results

| Metric | Value |
| --- | --- |
| Synthesized Cell Count | 1,786 |
| Total Cell Count (post-optimization) | 5,314 |
| Logic Depth | 13 |

### Cell Distribution (SKY130 standard cells)

| Cell Type | Count | Cell Type | Count |
| --- | --- | --- | --- |
| AND | 141 | XOR | 113 |
| NAND | 110 | XNOR | 38 |
| OR | 423 | MUX | 57 |
| NOR | 85 | DFF (flip-flops) | 81 |

Primary I/O: 385 inputs, 378 outputs.

# 7. OpenLane Physical Design Flow (RTL-to-GDSII Steps)

With RTL verified and synthesizable, the physical design flow is run as a single automated pipeline:

cd OpenLane
make mount
% ./flow.tcl -design i2c_uart_bridge

`make mount` drops you into the OpenLane Docker container; flow.tcl then runs every stage below in sequence.

💡 The `%` above is the OpenLane Tcl-shell prompt inside the container — do not type it yourself.

OpenLane executes the following stages automatically:

| # | Stage | What it does |
| --- | --- | --- |
| 1 | Logic Synthesis | Translates RTL into a gate-level netlist of SKY130 standard cells (Yosys). |
| 2 | Floorplanning & PDN | Sets die/core dimensions, places I/O pins, builds the power delivery network (VDD/VSS grid). |
| 3 | Placement | Global placement (minimizes wirelength) then detailed placement (legalizes, removes overlaps). |
| 4 | Clock Tree Synthesis | Builds a buffered clock tree (TritonCTS) to minimize skew and insertion delay to all flip-flops. |
| 5 | Routing | Global routing (assigns regions) then detailed routing (assigns tracks/metal layers/vias). |
| 6 | Static Timing Analysis | Verifies setup/hold timing across process corners without dynamic simulation. |
| 7 | Physical Verification | DRC (geometry rules), LVS (layout matches netlist), and antenna checks. |
| 8 | GDSII Generation | Exports the signed-off layout as the final top.gds for fabrication. |

# 8. Floorplanning Results

Floorplanning fixes the physical chip dimensions and utilization target before any cells are placed.

| Parameter | Value |
| --- | --- |
| Width | 203.78 µm |
| Height | 201.28 µm |
| Core Area | 41,016.84 µm² |
| Die Area | 0.04849 mm² |
| Floorplan Utilization Target | 50% |
| Final Utilization Achieved | 58.85% |

Figure 3.  —  Full-chip floorplan viewed in Magic — core, I/O ring, and standard-cell rows

Figure 4.  —  Floorplan detail in Magic showing the standard-cell placement grid at the transistor layer

To reproduce these Magic views directly:

magic -T sky130A.tech
% load top
% :box     
% :view

# 9. Placement Results

OpenROAD's global and detailed placement engines were run automatically inside the OpenLane flow.

| Metric | Value |
| --- | --- |
| Total Cells Placed | 5,314 |
| Overlap Violations | 0 |
| Placement Optimization | Completed successfully |

# 10. Clock Tree Synthesis (CTS) Results

Clock Tree Synthesis was performed using TritonCTS, which inserts a balanced tree of clock buffers so every flip-flop receives the clock edge at nearly the same time.

- Clock buffers inserted to balance load

- Clock repeaters added along long clock nets

- Fanout-balancing buffers to equalize sub-tree loads

Result: reduced clock skew, controlled insertion delay, and improved timing closure — reflected in the zero setup/hold violations reported in Section 11.

# 11. Routing Results

Routing connects every placed cell according to the netlist, first coarsely (global routing) and then with exact track/layer/via assignments (detailed routing).

| Metric | Value |
| --- | --- |
| Total Wire Length | 48,245 µm |
| Number of Vias | 13,948 |
| Routing Violations | 0 |

# 12. Static Timing Analysis (STA) Results

STA was run across Slow, Typical, and Fast process corners to mathematically validate all setup and hold timing paths.

| Parameter | Value |
| --- | --- |
| Clock Period Constraint | 10 ns |
| Target Frequency | 100 MHz |
| Suggested Clock Frequency | 99.8 MHz |
| Critical Path Delay | 1.39 ns |
| Worst Negative Slack (WNS) | 0 ns |
| Total Negative Slack (TNS) | 0 ns |
| Hold Violations | 0 |
| Setup Violations | 0 |
| Maximum Fanout Constraint | 10 |
| Fanout Violations | 25 |

💡 Fanout violations (25) are a design-quality warning, not a timing failure — some nets drive more loads than the recommended limit of 10. Setup and hold themselves are fully clean (0 violations), so the chip meets timing as-is; the fanout warning is a candidate for future optimization (see Section 16).

### Concept: Setup vs. Hold vs. Slack

Setup time checks that data arrives before the next clock edge; hold time checks that data doesn't change too soon after a clock edge. "Slack" is the margin between required and actual arrival time — 0 ns slack with 0 violations means every path meets its constraint with no margin to spare, i.e. the design is timing-closed exactly at its target frequency.

# 13. Power Analysis

Power was estimated after parasitic extraction, across the same three process corners used for STA.

| Process Corner | Internal (mW) | Switching (mW) | Leakage (mW) | Total (mW) |
| --- | --- | --- | --- | --- |
| Slow Corner | 0.00191 | 0.00066 | 0.0000158 | 0.002586 |
| Typical Corner | 0.00243 | 0.000856 | 0.000000301 | 0.003286 |
| Fast Corner | 0.00280 | 0.00102 | 0.000000313 | 0.003820 |

Total power stays under 4 µW across all corners — well within the budget of a low-power embedded communication peripheral.

# 14. DRC and LVS Verification

Physical verification confirms the layout is both manufacturable (DRC) and electrically equivalent to the synthesized netlist (LVS).

| Check | Tool(s) | Result |
| --- | --- | --- |
| Design Rule Check (DRC) | Magic | Pass — 0 violations |
| Design Rule Check (DRC) | KLayout | Pass — 0 violations |
| Layout vs. Schematic (LVS) | Netgen | Pass — 0 errors |
| Antenna Check | Magic / OpenROAD | Pass — 0 violations |
| XOR (layout diff check) | KLayout | Pass |

To reproduce DRC/LVS signoff checks manually inside the OpenLane container:

% run_magic_drc
% run_magic_spice_export
% run_lvs
% run_antenna_check

# 15. GDSII Generation and Visualization

The final stage exports the fully placed-and-routed, DRC/LVS-clean layout as a GDSII stream file — the industry-standard format sent to a foundry for fabrication.

# generated automatically by OpenLane at:
results/final/gds/top.gds

## 15.1 Viewing the Layout in KLayout

klayout results/final/gds/top.gds

Figure 5.  —  top.gds opened in KLayout with default layer colors, no custom layer map applied

Figure 6.  —  top.gds in KLayout with the SKY130 layer map loaded — metal, via, and diffusion layers colorized and net labels visible

Figure 7.  —  Zoomed KLayout view showing routed nets, standard-cell rows, and I/O pin labels (sda, scl, irq_n, clk)

## 15.2 Viewing / Verifying the Layout in Magic

magic -T sky130A.tech
% gds read top.gds
% load top

💡 Use Magic when you need to interactively probe layers, run DRC (`:drc check`), or extract SPICE for LVS. Use KLayout for fast viewing, layer-map based colorization, and independent DRC/XOR cross-checks against Magic's results.

# 16. Conclusion and Future Work

This project demonstrated a complete RTL-to-GDSII implementation of a UART-to-I2C bridge using an entirely open-source EDA toolchain, meeting timing closure at 100 MHz with zero setup and hold violations across all process corners.

### Achievements

- Functional RTL verification via Icarus Verilog / GTKWave

- Successful synthesis to SKY130 standard cells via Yosys

- Timing closure at 100 MHz (0 setup/hold violations, all corners)

- DRC-clean and LVS-clean layout (Magic, KLayout, Netgen)

- Signed-off GDSII generation, ready for tapeout submission

### Future Work

- Multi-master I2C support

- DMA integration for the FIFO buffer

- Low-power optimization and clock gating

- Multi-voltage domain implementation

- Reducing the 25 max-fanout warnings identified in Section 12

- Advanced protocol bridging, e.g. SPI-to-I2C or UART-to-SPI

# Appendix A: Full Command Reference

Every command used across this guide, in execution order, ready to copy-paste.

### A.1 One-time environment setup

sudo apt update
sudo apt install -y build-essential git python3 python3-pip \
    iverilog gtkwave yosys magic klayout docker.io
sudo systemctl enable --now docker
sudo usermod -aG docker $USER
newgrp docker
 
git clone https://github.com/The-OpenROAD-Project/OpenLane.git
cd OpenLane
make
make pdk
make test

### A.2 Simulation

cd designs/i2c_uart_bridge
iverilog -o sim.out src/*.v tb/*.v
vvp sim.out
gtkwave dump_top.vcd

### A.3 Standalone synthesis inspection (optional)

yosys -p "synth -top top; write_verilog synth_top.v; show -format dot -prefix yosys_show" src/*.v
xdot yosys_show.dot

### A.4 Full physical design flow

cd OpenLane
make mount
% ./flow.tcl -design i2c_uart_bridge

### A.5 Layout visualization and signoff

klayout results/final/gds/top.gds
 
magic -T sky130A.tech
% gds read top.gds
% load top
% run_magic_drc
% run_lvs

### A.6 Report extraction

cat ~/openlane_designs/i2c_uart_bridge/runs/RUN_<timestamp>/reports/metrics.csv
cat ~/openlane_designs/i2c_uart_bridge/runs/RUN_<timestamp>/reports/signoff/33-sta-rcx_nom/multi_corner_sta.summary.rpt
cat ~/openlane_designs/i2c_uart_bridge/runs/RUN_<timestamp>/reports/signoff/33-sta-rcx_nom/multi_corner_sta.power.rpt
cat ~/openlane_designs/i2c_uart_bridge/runs/RUN_<timestamp>/reports/signoff/33-sta-rcx_nom/multi_corner_sta.checks.rpt

💡 Replace RUN_<timestamp> with the actual run folder name OpenLane created (e.g. RUN_2026.07.10_04.58.13), visible under designs/i2c_uart_bridge/runs/ after flow.tcl completes.

# Appendix B: Glossary (for beginners)

| Term | Meaning |
| --- | --- |
| RTL | Register Transfer Level — behavioral Verilog/VHDL code describing logic in terms of registers and the operations between them. |
| Netlist | A list of standard-cell instances and the wires (nets) connecting them, produced by synthesis. |
| Standard cell | A pre-designed, pre-characterized logic gate (AND, DFF, etc.) from the PDK's cell library, used as a building block. |
| PDK | Process Design Kit — the foundry-specific rules, models, and cell libraries needed to design for a given fabrication process. |
| Floorplan | The physical outline (die/core size) and macro/pin placement defined before detailed placement. |
| Placement | Assigning an exact (x, y) location to every standard cell. |
| CTS | Clock Tree Synthesis — building the buffered network that distributes the clock signal with minimal skew. |
| Routing | Creating the physical metal wires and vias that realize the netlist's connections. |
| DRC | Design Rule Check — verifies the layout obeys the foundry's manufacturable geometry rules. |
| LVS | Layout Versus Schematic — verifies the layout's extracted netlist matches the synthesized netlist. |
| STA | Static Timing Analysis — checks setup/hold timing on every path without running a simulation. |
| GDSII | The binary layout file format sent to a foundry for fabrication. |
| Slack | Timing margin: (required arrival time) − (actual arrival time). Negative slack is a violation. |